- QMK Firmware Update Method

- Install the qmk_toolbox firmware update tool and open qmk_toolbox after installation.

- Click 'OPEN' and select the firmware for the specific keyboard.

- Switch the keyboard to wired mode (set the three-way switch to the middle position), then hold down the first key on the left side (usually the ESC key or press directly down on the rotary knob), and plug in the keyboard USB cable while holding down this key.

- Then click the 'Flash' button on the firmware update tool and wait until the update reaches 100% to complete the firmware update.