- QMK Firmware Update Method

- Install the QMK Toolbox firmware update tool. After installation, open QMK Toolbox.

- Click "OPEN" to select the corresponding firmware for your keyboard.

- Switch your keyboard to wired mode (set the three-position switch to the middle position), then hold down the first key on the left side (usually the ESC key or press down the rotary switch). After holding it down, connect the keyboard's USB cable.

- Then click the "Flash" button in the firmware update tool. Wait until the update reaches 100% to complete the firmware update.